#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <SPI.h>

#include <MFRC522.h>

#define BLYNK_TEMPLATE_ID "TMPL3de9Q8UHz"

#define BLYNK_TEMPLATE_NAME "Quickstart Template"

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET    -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

#define SS_PIN D8

#define RST_PIN D3

MFRC522 rfid(SS_PIN, RST_PIN);

// WiFi & Blynk Credentials

char auth[] = "TBQ0cT6E0ZEFLpUDNtezQckvD2eswiGV";

char ssid[] = "OnePlus Nord CE 2 Lite 5G";

char pass[] = "get lost";

struct Item {

byte uid[4];

const char* name;

float price;

};

Item items[] = {

{{0xFB, 0x5D, 0xE8, 0x3B}, "Jeans", 10},

{{0xEB, 0xBB, 0x9C, 0x3B}, "T-shirt", 8},

{{0xEB, 0x7D, 0x62, 0x3B}, "iphone", 50},

{{0xFB, 0x06, 0x88, 0x3B}, "Toothbrush", 3},

{{0xEB, 0x3F, 0xBE, 0x3B}, "Laptop", 100},

{{0x13, 0x1C, 0x79, 0x22}, "PANJAMI", 0}

};

const int numItems = sizeof(items) / sizeof(items[0]);

bool itemState[numItems];

#define BUTTON_PIN 2

float totalAmount = 0.0;

float walletBalance = 200.0;

bool lastButtonState = HIGH;

bool buttonPressed = false;

bool scanningEnabled = true;

unsigned long lastDebounceTime = 0;

unsigned long debounceDelay = 50;

BlynkTimer timer;

String billDetails = "";

bool compareUID(byte *uid, byte *storedUID) {

for (byte i = 0; i < 4; i++) {

if (uid[i] != storedUID[i]) return false;

}

return true;

}

void updateBlynkBill() {

String bill = "📜 **BILL SUMMARY** 📜\n";

bill += "---------------------------\n";

bill += billDetails;  // Add scanned items

bill += "---------------------------\n";

bill += "💰 **Total:** $" + String(totalAmount, 2) + "\n";

bill += "💳 **Wallet:** $" + String(walletBalance, 2) + "\n";

bill += "---------------------------\n";

Blynk.virtualWrite(V10, bill); // Send to Blynk Terminal

}

void setup() {

Serial.begin(115200);

// Reset item states

for (int i = 0; i < numItems; i++) {

itemState[i] = false;

}

// Reset total amount and wallet balance

totalAmount = 0.0;

walletBalance = 200.0; // Reset to initial wallet balance

billDetails = ""; // Clear bill details

Blynk.begin(auth, ssid, pass);

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("SSD1306 allocation failed");

while (1);

}

SPI.begin();

rfid.PCD_Init();

pinMode(BUTTON_PIN, INPUT_PULLUP);

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

display.setCursor(10, 20);

display.println("RFID Ready!");

display.display();

Serial.println("RFID & OLED Initialized");

// ✅ **Clear Blynk LCD Display (if using Virtual Pin V9)**

Blynk.virtualWrite(V9, " ");

// ✅ **Clear Blynk Terminal (if using Virtual Pin V10 for bill details)**

Blynk.virtualWrite(V10, " ");

// ✅ **Reset Wallet Balance in Blynk (Virtual Pin V8)**

Blynk.virtualWrite(V8, walletBalance);

// ✅ **Clear Last Scanned Item Price (Virtual Pin V5)**

Blynk.virtualWrite(V5, " ");

// ✅ **Reset Total Amount in Blynk (Optional)**

Blynk.virtualWrite(V6, totalAmount);

}

void loop() {

Blynk.run();

timer.run();

int buttonState = digitalRead(BUTTON_PIN);

if (buttonState == HIGH && lastButtonState == LOW && (millis() - lastDebounceTime) > debounceDelay) {

buttonPressed = true;

scanningEnabled = false;

lastDebounceTime = millis();

} else if (buttonState == LOW && lastButtonState == HIGH) {

buttonPressed = false;

scanningEnabled = true;

}

lastButtonState = buttonState;

if (scanningEnabled) {

if (!rfid.PICC_IsNewCardPresent() || !rfid.PICC_ReadCardSerial()) return;

Serial.print("RFID Tag UID: ");

for (byte i = 0; i < rfid.uid.size; i++) {

Serial.print(rfid.uid.uidByte[i], HEX);

Serial.print(" ");

}

Serial.println();

const char* itemName = "Unknown Item";

float itemPrice = 0.0;

bool itemFound = false;

int foundIndex = -1;

for (int i = 0; i < numItems; i++) {

if (compareUID(rfid.uid.uidByte, items[i].uid)) {

itemName = items[i].name;

itemPrice = items[i].price;

itemFound = true;

foundIndex = i;

break;

}

}

if (itemFound && foundIndex != -1) {

if (!itemState[foundIndex]) {

totalAmount += itemPrice;

walletBalance -= itemPrice;

itemState[foundIndex] = true;

billDetails += "🛒 " + String(itemName) + " - $" + String(itemPrice, 2) + "\n";

display.clearDisplay();

display.setTextSize(2);

display.setCursor(10, 10);

display.println("Added:");

display.setCursor(10, 35);

display.print(itemName);

display.display();

Serial.print("Added Item: ");

Serial.println(itemName);

} else {

totalAmount -= itemPrice;

walletBalance += itemPrice;

itemState[foundIndex] = false;

billDetails.replace("🛒 " + String(itemName) + " - $" + String(itemPrice, 2) + "\n", "❌ " + String(itemName) + " removed\n");

display.clearDisplay();

display.setTextSize(2);

display.setCursor(10, 10);

display.println("Removed:");

display.setCursor(10, 35);

display.print(itemName);

display.display();

Serial.print("Removed Item: ");

Serial.println(itemName);

}

Blynk.virtualWrite(V9, itemName);

Blynk.virtualWrite(V5, itemPrice);

Blynk.virtualWrite(V8, walletBalance);

updateBlynkBill();  // Send bill update to Blynk

}

rfid.PICC_HaltA();

}

if (buttonPressed) {

display.clearDisplay();

display.setTextSize(2);

display.setCursor(10, 10);

display.println("Total Amount");

display.setTextSize(3);

display.setCursor(10, 40);

display.print("$");

display.println(totalAmount, 2);

display.display();

updateBlynkBill(); // Show the bill in Blynk Terminal

delay(2000);

}

delay(500);

}